Task 1:  Autonomous Architectures

# Task Objective

The objective of this task is to select the most suitable autonomous sensing and onboard computing architecture for the given drone application. Five different sensor–computer combinations are compared based on compute capability, power consumption, mission suitability, weight, and cost.

A weighted decision matrix is then used to rank the architectures and select the configuration that provides the best overall balance for autonomous operation.

# Candidate Architectures

| Arch. | Sensor Configuration | Onboard Computer |
| --- | --- | --- |
| 1 | Forward-facing stereo — Custom/Waveshare IMX219-83 Stereo Camera | NVIDIA Jetson Orin Nano |
| 2 | Depth camera — OAK-D Lite + AI | Raspberry Pi 5, 8 GB |
| 3 | LiDAR — LiDAR d500 + Arducam IMX219 Wide Angle Camera | Raspberry Pi 5, 8 GB |
| 4 | Forward-facing stereo — Custom/Waveshare IMX219-83 Stereo Camera | Raspberry Pi 5, 8 GB |
| 5 | Depth camera — Intel RealSense D435i | NVIDIA Jetson Orin Nano |

# Mission Requirements

The drone has two different jobs.

## Job 1 — Find Odysseus

- Look over a large area

- Detect a human/person

- Work at useful distances

- Determine where the person is

## Job 2 — Don't crash while doing it

- Detect obstacles

- Estimate distance

- Navigate autonomously

- Maintain stable flight

# Decision Criteria & Weights

| Criterion | Weight |
| --- | --- |
| Use-case suitability | 35% |
| Power | 20% |
| Weight | 20% |
| Compute | 15% |
| Cost | 10% |
| Total | 100% |

# Onboard Computers

## Raspberry Pi 5, 8GB

- General-purpose SBC, not specifically designed for AI.

- CPU: Quad-core Cortex-A76, 2.4 GHz.

- RAM: 8 GB.

- Good for navigation, sensor handling, ROS and moderate computer vision.

- No dedicated AI/Tensor hardware → weaker for heavy object detection and stereo processing.

- Lower power, lighter and cheaper than Jetson.

- Works especially well with OAK-D Lite, because OAK-D performs depth + AI processing onboard.

- Best advantage: efficiency/cost.

- Main weakness: limited AI compute.

## NVIDIA Jetson Orin Nano, 8GB

- AI/robotics-focused computer.

- CPU: 6-core Cortex-A78AE.

- NVIDIA Ampere GPU + Tensor Cores.

- Excellent for person detection, tracking, stereo vision, SLAM and sensor fusion.

- Supports CUDA and TensorRT for accelerated AI.

- Much more AI compute than Pi 5.

- Higher power consumption, weight/cooling requirement and cost.

- Best when the computer itself needs to perform heavy vision processing.

- Best advantage: AI/computer-vision performance.

- Main weakness: power + cost + system weight.

# Architecture Comparison

| Arch. | Sensor + Computer | Compute | Sensing / Range | Power | Weight | Relative Cost |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Stereo IMX219 + Jetson Orin Nano | 67 TOPS | Stereo vision, 83° diagonal FOV | High | ~198 g | Moderate–High |
| 2 | OAK-D Lite + Raspberry Pi 5 | 4 TOPS on camera + Pi compute | Depth camera, ~0.4–8 m ideal range | Low–Moderate | ~107 g | Moderate–High |
| 3 | D500 LiDAR + Arducam + Raspberry Pi 5 | Pi compute | 360° LiDAR, 0.03–12 m | Low | ~99 g | Moderate |
| 4 | Stereo IMX219 + Raspberry Pi 5 | Pi compute | Stereo vision, 83° diagonal FOV | Low–Moderate | ~69 g | Lowest |
| 5 | RealSense D435i + Jetson Orin Nano | 67 TOPS | Depth camera, ~0.3–3 m ideal range | High | ~250 g | Highest |

Note: Cost is compared relatively because market prices vary. Power and weight refer only to the sensing and computing hardware, not the complete drone.

# Key Observations

### Architecture 1 Stereo IMX219 + Jetson Orin Nano

- Two forward-facing cameras forming a stereo pair.

- Both cameras observe the same scene from slightly different positions.

- Jetson calculates disparity between the two images.

Z=Depth, B= separation between two cameras, d= disparity, f= focal length

- From disparity, it estimates depth:  (Working of a Stereo Camera)
Z=fBd

- The same Jetson can run AI models for person detection, tracking and navigation.

- However, stereo depth becomes increasingly inaccurate as distance increases.

- Water is also a difficult stereo environment because reflections, glare, repetitive/low-texture regions and waves can make feature matching harder.

### Architecture 2 OAK-D Lite + Raspberry Pi 5

- The OAK-D Lite itself performs vision/depth/AI processing. Therefore: Pi 5 + OAK-D processing is much more capable for this workload than simply saying "Raspberry Pi = weak."

- OAK-D is designed as an efficient edge-vision device.

- Pi 5 generally requires less power than a heavily loaded Jetson setup.

- Onboard processing means you aren't brute forcing every vision operation on the Pi.

## Architecture 3 D500 LiDAR + IMX219 + Raspberry Pi 5

- D500 provides 360° horizontal ranging; camera provides visual/person detection.

- LiDAR is good for obstacle avoidance and navigation, but is only 2D.

- Pi must handle most of the camera-based AI processing.

- Low power and lightweight.

- Key point: Excellent obstacle sensing, but weaker for AI-heavy vision.

## Architecture 4 Stereo IMX219 + Raspberry Pi 5

- Stereo cameras provide depth, but the Pi must calculate stereo depth itself.

- Pi must also handle person detection + navigation, creating a heavy computational load.

- Low power, lightweight and cheapest.

- Key point: Efficient and cheap, but compute-limited.

## Architecture 5 D435i + Jetson Orin Nano

- D435i provides RGB + stereo depth + IMU, with dedicated depth processing.

- Jetson provides excellent AI capability for detection, tracking and navigation.

- Most computationally capable architecture.

- Higher power, weight and cost; D435i depth is mainly useful at relatively short ranges.

- Key point: Maximum capability, but worst cost trade-off.

# Weighted Decision Matrix

Each candidate architecture was scored 1–10 against five weighted criteria. The weighted score determines the overall ranking.

| Architecture | Use-case suitability 30% | Compute 25% | Power 20% | Weight 15% | Cost 10% | Weighted Score /10 | Rank |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Architecture 1 Stereo IMX219 + Jetson Orin Nano | 8 | 10 | 4 | 6 | 6 | 7.20 | 4 |
| Architecture 2 OAK-D Lite + Raspberry Pi 5 | 9 | 8 | 8 | 8 | 6 | 8.10 | 1 |
| Architecture 3 D500 LiDAR + Arducam + Raspberry Pi 5 | 7 | 5 | 9 | 9 | 8 | 7.30 | 3 |
| Architecture 4 Stereo IMX219 + Raspberry Pi 5 | 6 | 5 | 9 | 10 | 10 | 7.35 | 2 |
| Architecture 5 RealSense D435i + Jetson Orin Nano | 9 | 10 | 4 | 5 | 4 | 7.15 | 5 |

# Conclusion

Based on the weighted comparison, Architecture 2 — OAK-D Lite + Raspberry Pi 5 is the best overall choice for the mission, with a final score of 8.10/10. It provides a good balance of person detection, depth perception, onboard AI processing, low power consumption, and low weight. The OAK-D Lite handles much of the vision and depth processing itself, so the Raspberry Pi 5 does not need to carry the entire computational load. Although Architecture 5 offers much higher compute capability, its higher power consumption, weight, and cost make it less suitable for a weight- and endurance-constrained drone. Therefore, Architecture 2 gives the best overall trade-off for the search-and-rescue mission.

# Bonus – Camera Orientation

- I would keep the main camera facing forward but slightly tilted downward.

- The forward direction helps the drone see where it is going and avoid obstacles, while the downward tilt allows it to see more of the sea surface while searching for the person.

- If possible, an additional downward-facing camera can be added, which would make searching directly below the drone easier.

- So, forward + slightly downward would be a good balance between navigation and searching.

# Bonus – Alternate Build

- The OAK-D Lite provides RGB imaging, stereo depth and onboard AI/depth processing.

- The Jetson Orin Nano provides much higher processing power for person detection, tracking and autonomous navigation.

- Since the OAK-D already handles some vision processing, the Jetson can focus more on the heavier AI and navigation tasks.

- Compared with D435i + Jetson, this setup can be slightly lighter and cheaper, while still giving strong perception and compute capability.

- The main drawback is that both OAK-D and Jetson have AI-processing capability, so some of the compute may be underused if the workload is not divided properly.

This alternate build is suitable when higher AI performance is preferred over minimum power consumption. This build gives a good combination of efficient depth sensing from OAK-D Lite and high AI compute from Jetson Orin Nano, making it suitable for a search-and-rescue drone where both perception and processing power are important.

Note on AI Usage:

AI was used only for document design and presentation. The research, technical content, calculations, and final conclusions were done and verified independently.