AVIONICS TASK REPORT · TASK 03

Propulsion System

| MOTOR Holybro S500 V2 2216-920KV | PROPELLER 1045 10 × 4.5 inch | MAX MASS 2.5 kg drone, fully loaded | ENDURANCE > 12 min required, hover |
| --- | --- | --- | --- |

# 01  Component Specifications

TABLE 1 — HOLYBRO S500 V2 2216-920KV

| Specification | Value |
| --- | --- |
| Model | S500 V2 |
| KV Rating | 920 KV |
| Motor Diameter | 28 mm |
| Motor Weight | 63 g |
| Supported Battery | 3S–4S LiPo |
| Stator Diameter | 22 mm |
| Wire Length | 60 mm |
| Compatible Propeller | 10 inch |

TABLE 2 — PRO-RANGE 1045 PROPELLER

| Specification | Value |
| --- | --- |
| Propeller Size | 10 × 4.5 inch |
| Diameter | 10 inch |
| Pitch | 4.5 inch |
| Material | ABS |
| Colour | Black |
| Weight | 22 g |
| Shaft Diameter | 9 mm |
| Total Length | 250 mm |
| Rotation | 1 CW + 1 CCW |

# 02  Battery Calculation

Assumed total drone mass · no. of motors m = 2 kg = 2000 g n = 4

Thrust required per motor at hover T = 2000 / 4

T = 500 g

Motor current, from motor test data 470 g thrust ≈ 3.82 A therefore, for nearly 500 g: Imotor ≈ 4 A

Current drawn by 4 motors + electronics Imotors = 4 × 4 = 16 A Ielectronics ≈ 2 A Itotal = 16 + 2

Itotal ≈ 18 A

Required endurance t = 12 min = 12 / 60 hr = 0.2 hr

Usable capacity required C = I × t = 18 × 0.2 = 3.6 Ah

Taking usable battery capacity as 80% Cbattery = 3.6 / 0.8 = 4.5 Ah = 4500 mAh

C = 5000 mAh (nearest suitable capacity)

Battery voltage, for 4S LiPo V = 4 × 3.7

V = 14.8 V

Battery selected — 4S 5000 mAh LiPo

# 03  Endurance Check

Usable capacity C = 5 × 0.8 = 4 Ah

Estimated endurance t = (C / I) × 60 = (4 / 18) × 60

t ≈ 13.3 min

since  13.3 min  >  12 min

Endurance requirement satisfied

# 04  C-Rating Verification

Maximum discharge current, for 5000 mAh = 5 Ah at 20C Imax = C × C-rating = 5 × 20

Imax = 100 A

Maximum combined rating of four 20A ESCs I = 4 × 20 = 80 A

since  100 A  >  80 A

20C battery is sufficient

# 05  ESC Selection

ESC rating per motor · number of ESCs required ESC per motor = 20 A no. of ESCs = 4

4 × 20A BLHeli-S ESCs

20A BLHeli-S ESC is selected because it is the ESC recommended and used by Holybro with the given 2216-920KV motor and 1045 propeller combination. Its current rating is sufficient for the motor while avoiding unnecessary extra weight from an oversized ESC.

# 06  Final Selection

TABLE 3 — FINAL PROPULSION SYSTEM

| Specification | Value |
| --- | --- |
| Motor | Holybro S500 V2 2216-920KV |
| Propeller | 1045 (10 × 4.5) |
| Battery | 4S 5000 mAh 20C+ LiPo |
| ESC | 4 × 20A BLHeli-S |
| Estimated Endurance | ≈ 13.3 min |
| Weight Limit | < 2.5 kg |

# 07  Conclusion

The selected 4S 5000 mAh battery and 20A BLHeli-S ESCs are compatible with the given motor-propeller setup. The estimated endurance is above 12 minutes, so the propulsion system satisfies the main design requirement.

# 08  Bonus — Alternative Motor

For an alternative motor, I looked for a motor that could:

▸  work with a 4S battery

▸  be used with a propeller close to our 10-inch propeller

▸  give around 500 g thrust, which is close to our required hover thrust

▸  give the same thrust using less power

The given Holybro 2216-920KV motor gives approximately:

Holybro 2216-920KV — baseline performance Thrust = 470.2 g Power = 61.51 W

Efficiency, therefore Efficiency = 470.2 / 61.51

Efficiency ≈ 7.64 g/W

A suitable alternative is the T-Motor MN2212 V2 KV780. At 14.8 V with a 9.5 × 4.5 propeller, it gives:

T-Motor MN2212 V2 KV780 — performance Thrust = 476 g Power = 59.2 W

Efficiency, therefore Efficiency = 476 / 59.2

Efficiency ≈ 8.04 g/W

TABLE 4 — MOTOR COMPARISON

| Motor | Thrust | Power | Efficiency |
| --- | --- | --- | --- |
| Holybro 2216-920KV | 470.2 g | 61.51 W | 7.64 g/W |
| T-Motor MN2212 KV780 | 476 g | 59.2 W | 8.04 g/W |

## Conclusion

The T-Motor MN2212 KV780 can be considered as a suitable alternative because it produces almost the same thrust while consuming slightly less power. Its thrust-to-power efficiency is also better than the given motor. It supports 2S–4S LiPo batteries and weighs about 65 g including cables, so it remains close to the size and weight of the original motor.

Note on AI Usage:

AI was used only for document design and presentation. The research, technical content, calculations, and final conclusions were done and verified independently.